✅ Day 10 Practical Lab

1️⃣ Check Your Network Interface

Run:

ip a

Find:

- Interface name → enp0s3

- Your IP address

Example:

inet 192.168.1.93/24

Write this in notes.

2️⃣ Check Default Gateway

Run:

ip route

Example output:

default via 192.168.1.1 dev enp0s3

Meaning:

| Item | Meaning |
| --- | --- |
| default | route to internet |
| 192.168.1.1 | router / gateway |
| enp0s3 | interface |

Write in notes:

Default Gateway = 192.168.1.1

3️⃣ Analyze Packet Flow

When you run:

ping google.com

The packet flow is:

VM
↓
Default Gateway (Router)
↓
ISP Network
↓
Internet Backbone
↓
Google Server

Write this exactly in your notes.

4️⃣ Run Traceroute

Run:

traceroute google.com

Example:

1 192.168.1.1
2 ISP network
3 ISP backbone
4 Internet backbone
5 Google server

Understand:

| Hop | Meaning |
| --- | --- |
| 1 | Your router |
| 2 | ISP |
| 3+ | Internet |

Your screenshot already shows this correctly.

5️⃣ Install SSH (if not installed)

Check:

rpm -qa | grep openssh-server

If missing install:

sudo dnf install openssh-server -y

6️⃣ Start SSH Server

Run:

sudo systemctl start sshd

Enable at boot:

sudo systemctl enable sshd

Check status:

systemctl status sshd

Expected:

active (running)

7️⃣ Verify SSH Port

Run:

ss -tulnp

Look for:

tcp LISTEN 0 128 0.0.0.0:22

Meaning:

| Field | Meaning |
| --- | --- |
| tcp | protocol |
| LISTEN | service waiting |
| 22 | SSH port |

8️⃣ Ping Second VM

From VM1

ping VM2_IP

Example:

ping 192.168.56.102

Expected:

64 bytes from 192.168.56.102

9️⃣ SSH Into VM2

Run:

ssh username@VM2_IP

Example:

ssh aditya@192.168.56.102

If successful:

aditya@server:~$

You are now logged into the second VM.

📄 What Your Notes Should Contain (Day 10)

Write these:

Packet Flow

VM → Gateway → ISP → Internet → Server

Important Commands

ip a
ip route
traceroute
ssh
ss -tulnp

SSH Port

22

Default Gateway Meaning

Router that sends traffic outside local network.

✅ Day 10 Expected Output

You should have:

✔ traceroute analyzed
✔ packet flow written
✔ SSH running
✔ SSH port verified
✔ connectivity between VMs

1️⃣ Check Your Network Interface

2️⃣ Check Default Gateway

3️⃣ Analyze Packet Flow

4️⃣ Run Traceroute

5️⃣ Install SSH (if not installed)

6️⃣ Start SSH Server

7️⃣ Verify SSH Port

8️⃣ Ping Second VM

8️⃣ Ping Second VM