✅ Day 3 Plan

🔵 Part 1 — Subnetting Mastery (45–60 min)

Today focus on understanding block size and subnet jumps.

Step 1: Quick Revision (10 min)

Remember this table:

| CIDR | Hosts | Block Size |
| --- | --- | --- |
| /24 | 254 | 256 |
| /25 | 126 | 128 |
| /26 | 62 | 64 |
| /27 | 30 | 32 |
| /28 | 14 | 16 |
| /29 | 6 | 8 |
| /30 | 2 | 4 |

Understand how networks increment.

Example:

192.168.1.0/27

Block size = 32

Networks:

192.168.1.0
192.168.1.32
192.168.1.64
192.168.1.96
192.168.1.128

Step 2: Solve 20 Problems

Example questions:

- 192.168.5.0/27

- 192.168.5.64/27

- 10.0.0.128/26

- 172.16.10.0/28

- 192.168.2.0/29

For each find:

- Network ID

- Broadcast

- Usable range

- Total hosts

Target: < 90 seconds per problem

🔵 Part 2 — Linux Networking Commands (60–70 min)

Today you must become comfortable with network troubleshooting commands.

Open your Linux VM.

Step 1 — Check Network Interfaces

Run:

ip addr

Understand:

- interface name (eth0 / enp0s3)

- IP address

- subnet mask

Step 2 — Check Routing Table

Run:

ip route

Example output:

default via 192.168.1.1 dev enp0s3
192.168.1.0/24 dev enp0s3 proto kernel

Understand:

- default route

- local network

Write in notes:
What each line means.

Step 3 — Connectivity Tests

Run:

ping 8.8.8.8

Then:

ping google.com

Understand difference:

- IP ping tests network connectivity

- domain ping tests DNS + network

Step 4 — DNS Lookup

Run:

nslookup google.com

or

dig google.com

Observe:

- DNS server

- resolved IP

Step 5 — Trace Packet Path

Run:

traceroute google.com

See how packets travel through routers.

This is real networking visibility.

🔵 Part 3 — Mini Experiment (15 min)

Break DNS intentionally.

Edit:

/etc/resolv.conf

Change DNS to wrong value.

Now test:

ping google.com

Then test:

ping 8.8.8.8

Observe difference.

Fix DNS afterward.

🎯 Day 3 Output

By end of today you must have:

✅ 20 subnet problems solved
✅ Understand block size logic
✅ Know these commands:

ip addr
ip route
ping
nslookup
dig
traceroute

✅ Understand difference between:

- routing issue

- DNS issue

Part 2

Part 3

- What happens if DNS is wrong? I can still reach the internet by IP (numbers), but I cannot use website names.

- Difference between ping IP vs ping domain? Pinging an IP checks the physical road/gateway. Pinging a Domain checks the road AND the naming service (DNS)

1. The "Command Toolkit" Summary

These are the 6 essential tools you used today to see your network.

| Command | Simple Purpose | What to look for |
| --- | --- | --- |
| ip addr | My Identity | Look for enp0s3 and your IP (e.g., 192.168.0.50). |
| ip route | The Exit Door | Look for default via. This is your Gateway. |
| ping | The Heartbeat | Checks if a specific "house" (IP) is alive. |
| nslookup | The Phonebook | Asks the DNS: "What is the IP for this name?" |
| dig | The Deep Lookup | Same as nslookup, but gives more technical details. |
| traceroute | The GPS Path | Shows every "hop" (router) your data hits. |

2. Routing vs. DNS (The "Broken" Scenarios)

This is the most important "Engineer" knowledge you gained from your experiments.

A. Routing Issue (Wrong Gateway)

- The Problem: Your "car" (data) can't find the exit ramp to the highway.

- Result: You are trapped.

- Test: Both ping 8.8.8.8 (IP) and ping google.com (Name) FAIL.

B. DNS Issue (Wrong Nameserver)

- The Problem: Your "car" can drive on the highway, but your "GPS" is broken.

- Result: You can reach the internet only if you know the number (IP).

- Test: ping 8.8.8.8 (IP) PASSES ✅, but ping google.com (Name) FAILS ❌.